Sensitive instrumental method for quantitative determination of high-brominated flame retardants in human serum samples.

Cristina Grande1, Argelia Castaño1, Juan José Ramos1*

1 Environmental Toxicology, National Center for Environmental Health (CNSA), Instituto de Salud Carlos III, 28220, Madrid, Spain.

* Corresponding author: Dr. Juan José Ramos Tel.: +0034 918223896, ORCID: 0000-0001-9079-6740, E-mail address: jjramos@isciii.es

Cristina Grande, ORCID: 0000-0001-6300-2298

Argelia Castaño, ORCID: 0000-0003-4152-7119

Supplementary Materials. Tables

Supplementary material, Table 1: Ions selected for analytes quantification / confirmation.

| Compound | Acronym | CAS number | Quantification/ confirmation ions |
| --- | --- | --- | --- |
| Decabromo[13C12]diphenyl ether (IS) | 13C12-BDE-209 | 562099-68-7 | 492.6 / 494.6 |
| Decabromodiphenyl ether | BDE-209 | 1163-19-5 | 486.6 / 484.6 |
| Decabromodiphenylethane | DBDPE | 84852-53-9 | 79/81 |
| Tetrabromobisphenol A bis(2,3-dibromopropyl ether | TBBPA-DBPE | 21850-44-2 | 79/81 |

Supplementary material, Table 2: Standard calibration and matrix-matched calibration for BDE-209 quantification in human serum samples from interlaboratory studies.

| Quantification method | Sample 1 | Sample 2 |
| --- | --- | --- |
| Assigned value (Acceptable range) | 1.18 ng/mL (0.418 - 1.94 ng/mL) | 0.20 ng/mL (0.013 – 0.393 ng/mL) |
| Matrix-matched calibration | 1.33 ng/mL | 0.23 ng/mL |
| % RSD | 5.6 | 2.5 |
| Recovery | 113 % | 115 % |
| Standard calibration | 1.31 ng/mL | 0.26 ng/mL |
| % RSD | 5.2 | 2.2 |
| Recovery | 111 % | 130 % |

Supplementary material, Table 3. Demographic details of population included in this study. Random subsample obtained from the Bioambiet.es study

| Variable | Levels | Participants |
| --- | --- | --- |
| Gender | Males | 22 |
|  | Females | 18 |
| Age groups | 18-29 | 8 |
|  | 30-39 | 15 |
|  | 40-49 | 10 |
|  | 50-65 | 7 |
| Working Sector | Services | 27 |
|  | Others | 13 |
| Seasonality | Jan-Mar | 7 |
|  | Apr-Jun | 16 |
|  | Jul-Sep | 6 |
|  | Oct-Dic | 11 |
| Environment | Urban | 31 |
|  | Residential | 7 |
|  | Countryside | 2 |

Supplementary Materials. Figures

Supplementary Material, Figure. 1. GC-NCI-MS chromatograms obtained from a standard solution (1 ng/mL) on a 15 m DB-5MS column (0.25 mm id. 0.10 µm film thickness). (A) First injection after column installation, (B) after 200 injections.

Supplementary Material, Figure. 2. GC-NCI-MS chromatograms obtained after applying the developed method from (A) serum rabbit non spiked and (B) water reagent blank.

Supplementary Material, Figure 3. Total ion chromatogram (GC-NCI-MS) of 1 mL human serum sample after applying the developed method.