  - Supplementary Methods 1.

  - Detailed Methodology of Proteomic Analysis

  - Serum EV samples processe

  - d for protein identification by LC-MS/MS using a DDA method and quantification by LC-MS/MS using a DIA-SWATH method

The EV proteins were loaded on a 10% SDS-PAGE gel. The run was stopped as soon as the front had penetrated 3 mm into the resolving gel [1–4]. The protein band was detected by Sypro-Ruby fluorescent staining (Lonza, Switzerland), excised, and processed for in-gel, manual tryptic digestion, as described elsewhere [5]. Peptides were extracted by performing three 20-min incubations in 40 μL of 60% acetonitrile dissolved in 0.5% HCOOH. The resulting peptide extracts were pooled, concentrated in a SpeedVac, and stored at −20 °C.

Mass spectrometric analysis

We separated 4 μl (4 μg) of digested peptides of all individual samples, employing reversed-phase chromatography. A gradient was created using a micro liquid chromatography system (Eksigent Technologies nanoLC 400, SCIEX) coupled to a high-speed Triple TOF 6600 mass spectrometer (SCIEX) with a micro flow source. The chosen analytical column was a silica-based reversed-phase column Chrom XP C18 150 × 0.30 mm, 3-mm particle size and 120 Å pore size (Eksigent, SCIEX). The trap column was a YMC-Triart C18 (YMC Technologies,  Teknokroma) with a 3-mm particle size and a 120 Å pore size, switched on-line with the analytical column. The loading pump delivered a solution of 0.1% formic acid in water at 10 µl/min. The micro-pump generated a flow-rate of 5 µl/min and was operated under gradient elution conditions, using 0.1% formic acid in water as mobile phase A, and 0.1% formic acid in acetonitrile as mobile phase B. Peptides were separated by a 90-minute gradient ranging from 2% to 90% mobile phase B (mobile phase A: 2% acetonitrile, 0.1% formic acid; mobile phase B: 100% acetonitrile, 0.1% formic acid) [1–4,6–9].

Data acquisition was performed with a TripleTOF 6600 System (SCIEX, Foster City, CA, USA), using a data-dependent acquisition (DDA) method. Source and interface conditions were as follows: ion spray voltage floating (ISVF) 5500 V, curtain gas (CUR) 25, collision energy (CE) 10, and ion source gas 1 (GS1) 25. The instrument was operated with Analyst TF 1.7.1 software (SCIEX, USA). Switching criteria was set to ions greater than mass to charge ratio (m/z) 350 and smaller than m/z 1400 with a charge state of 2-5, mass tolerance 250 ppm, and an abundance threshold of more than 200 counts (cps). Former target ions were excluded for 15 s. The instrument was automatically calibrated every 4 hours using tryptic peptides from PepCalMix as the external calibrant.

Data Analysis

After MS/MS analysis, data files were processed with ProteinPilotTM 5.0.1 software from SCIEX, which uses the algorithm ParagonTM for database search and ProgroupTM for data grouping. Data were searched using a human-specific Uniprot database (UniProt release 2020_01 Published on February 26, 2020; 20365 human proteins). The false discovery rate was determined using a nonlinear fitting method displaying only those results that reported a 1% global false discovery rate or better [10].

Protein quantification by SWATH (sequential window acquisition of all theoretical mass spectra)

Creation of the spectral library

To construct the MS/MS spectral libraries, the peptide solutions were analyzed by a shotgun DDA approach by micro-LC-MS/MS. To get a good representation of the peptides and proteins present in all samples, pooled  samples from each group were prepared using equal mixtures of the original samples. We separated 4 μL (4μg) of each pool into a micro-LC system Ekspert nLC425 (Eksigen, Dublin, CA, USA) using a column Chrom XP C18 150 × 0.30 mm, 3 mm particle size and 120 Å pore size (Eksigen, SCIEX), at a flow rate of 5 μL/min. Water and ACN, both containing 0.1% formic acid, were used as solvents A and B, respectively. The gradient run consisted of 5% to 95% B for 30 min, 90% B for 5 min, and finally 5% B for 5 min for column equilibration, for a total run time of 40 min. When the peptides were eluted, they were directly injected into a hybrid quadrupole-TOF mass spectrometer Triple TOF 6600 (SCIEX, Redwood City, CA, USA) operated with a DDA system in positive ion mode. A micro source (SCIEX) was used for the interface between microLC and MS, with an application of 2600 V voltage. The acquisition mode consisted of a 250 ms survey MS scan from 400 to 1250 m/z followed by an MS/MS scan from 100 to 1500 m/z (25 ms acquisition time) of the top 65 precursor ions from the survey scan, for a total cycle time of 2.8 s. The fragmented precursors were then added to a dynamic exclusion list for 15 s; any singly charged ions were excluded from the MS/MS analysis.

The peptide and protein identifications were performed with Protein Pilot software (version 5.0.1, SCIEX), searched with a human-specific Uniprot database (UniProt release 2020_01 Published on February 26, 2020; 20365 human proteins), specifying iodoacetamide as Cys alkylation. The false discovery rate (FDR) was set to <1% for both peptides and proteins. The MS/MS spectra of the identified peptides were then used to generate the spectral library for SWATH. To generate this library, all the pooled runs were submitted to ProteinPilot to obtain the SWATH library. Only peptides with a confidence score above 99% (as obtained from the Protein Pilot database search) were included in the spectral library.

Relative quantification by SWATH acquisition

SWATH–MS acquisition was performed with a TripleTOF® 6600 LC-MS/MS system (SCIEX). Samples were analyzed using a data-independent acquisition (DIA) method (30 total samples). Each 4-μL sample (from a mg/ml solution) was analyzed with the LC-MS equipment and LC gradient described above for building the spectral library, but instead employing the SWATH-MS acquisition method. This method involved repeating a cycle that consisted of the acquisition of 65 TOF MS/MS scans (400 to 1500 m/z, high sensitivity mode, 50 ms acquisition time) of overlapping sequential precursor isolation windows of variable width (1 m/z overlap), covering the 400 to 1250 m/z mass range with a previous TOF MS scan (400 to 1500 m/z, 50 ms acquisition time) for each cycle. Total cycle time was 6.3 s. For each sample set, the width of the 100 variable windows was optimized according to the ion density found in the DDA runs using a SWATH variable window calculator worksheet from SCIEX [11–13].

Data analysis

The targeted data extraction of the fragment ion chromatogram traces from the SWATH runs was performed by PeakView (version 2.2) using the SWATH Acquisition MicroApp (version 2.0). This application processed the data using the spectral library created from the shotgun data (DDA). Up to 10 peptides per protein and 7 fragments per peptide were selected, based on signal intensity; any shared and modified peptides were excluded from processing. Five-minute windows and 30-ppm widths were used to extract the ion chromatograms; SWATH quantitation was attempted for all proteins in the ion library that were identified by ProteinPilot with an FDR below 1%. The retention times of the peptides that were selected for each protein were realigned in each run according to the internal retention time peptides corresponding to the various proteins identified in each sample and eluted along the whole-time axis.  The extracted ion chromatograms were then generated for each selected fragment ion; the peak areas for the peptides were obtained by summing the peak areas from the corresponding fragment ions. PeakView computed an FDR and a score for each assigned peptide according to the chromatographic and spectral components; only peptides with an FDR below 1% were used for protein quantitation. Protein quantitation was calculated by adding the peak areas of the corresponding peptides.

The integrated peak areas (processed. mrkvw files from PeakView) were directly exported to MarkerView software (SCIEX) for relative quantitative analysis. The export generated 3 files containing quantitative information about individual ions, the summed intensity of different ions for a particular peptide, and the summed intensity of various peptides for a particular protein. MarkerView had been used for analysis of SWATH-MS data reported in other proteomics studies [14–17] because of its data-independent quantitation method. MarkerView uses processing algorithms that accurately find chromatographic and spectral peaks direct from the raw SWATH data. Data alignment by MarkerView compensates for minor variations in both mass and retention time values, ensuring that identical compounds in different samples are accurately compared with one another. To control for possible uneven sample loss across the various samples during the sample preparation process, we performed a global normalization based on the total sum of all the peak areas extracted from all the peptides and transitions across the replicates of each sample. Unsupervised multivariate statistical analysis using principal component analysis was performed to compare the data across the samples. The average MS peak area of each protein was derived from the replicates of the SWATH-MS of each sample followed by a Student’s t-test analysis using MarkerView software for comparison among the samples based on the averaged area sums of all the transitions derived for each protein. The t-test indicates how well each variable distinguishes the 2 groups, reported as a p-value. For each library, its set of differentially expressed proteins (p-value <0.05) with a 2-fold increase or decrease was selected.

References:

1. 	Laso-García F, Casado-Fernández L, Piniella D, et al. Circulating extracellular vesicles promote recovery in a preclinical model of intracerebral hemorrhage. Mol Ther Nucleic Acids. 2023;32:247-262. doi:10.1016/j.omtn.2023.03.006

2. 	Laso-García F, Piniella D, Gómez-de Frutos MC, et al. Protein content of blood-derived extracellular vesicles: An approach to the pathophysiology of cerebral hemorrhage. Front Cell Neurosci. 2022;16:1058546. doi:10.3389/fncel.2022.1058546

3. 	Gonzalez-Rellan MJ, Fernández U, Parracho T, et al. Neddylation of phosphoenolpyruvate carboxykinase 1 controls glucose metabolism. Cell Metab. Published online July 28, 2023. doi:10.1016/j.cmet.2023.07.003

4. 	Pereira-Veiga T, Bravo S, Gómez-Tato A, et al. Red blood cells protein profile is modified in breast cancer patients. Molecular & Cellular Proteomics. 2022;21(12):100435. doi:10.1016/j.mcpro.2022.100435

5. 	Shevchenko A, Wilm M, Vorm O, et al. A strategy for identifying gel-separated proteins in sequence databases by MS alone. Biochem Soc Trans. 1996;24(3):893-896.

6. 	Alonso-Sampedro M, Feás X, Bravo SB, Chantada-Vázquez MP, Vidal C. Proteomics of Vespa velutina nigrithorax Venom Sac Queens and Workers: A Quantitative SWATH-MS Analysis. Toxins (Basel). 2023;15(4). doi:10.3390/toxins15040266

7. 	Torres Iglesias G, Fernández-Fournier M, Botella L, et al. Brain and immune system-derived extracellular vesicles mediate regulation of complement system, extracellular matrix remodeling, brain repair and antigen tolerance in Multiple sclerosis. Brain Behav Immun. 2023;113:44-55. doi:10.1016/j.bbi.2023.06.025

8. 	Camino T, Lago-Baameiro N, Sueiro A, et al. Brown Adipose Tissue Sheds Extracellular Vesicles That Carry Potential Biomarkers of Metabolic and Thermogenesis Activity Which Are Affected by High Fat Diet Intervention. Int J Mol Sci. 2022;23(18). doi:10.3390/IJMS231810826

9. 	Camino T, Lago-Baameiro N, Bravo SB, et al. Human obese white adipose tissue sheds depot-specific extracellular vesicles and reveals candidate biomarkers for monitoring obesity and its comorbidities. Transl Res. 2022;239:85-102. doi:10.1016/J.TRSL.2021.01.006

10. 	Shilov I V., Seymour SL, Patel AA, et al. The Paragon Algorithm, a Next Generation Search Engine That Uses Sequence Temperature Values and Feature Probabilities to Identify Peptides from Tandem Mass Spectra. Molecular & Cellular Proteomics. 2007;6(9):1638-1655. doi:10.1074/mcp.T600050-MCP200

11. 	López-Valverde L, Vázquez-Mosquera ME, Colón-Mejeras C, et al. Characterization of the plasma proteomic profile of Fabry disease: Potential sex- and clinical phenotype-specific biomarkers. Transl Res. 2024;269:47-63. doi:10.1016/J.TRSL.2024.02.006

12. 	Fondevila MF, Novoa E, Gonzalez-Rellan MJ, et al. p63 controls metabolic activation of hepatic stellate cells and fibrosis via an HER2-ACC1 pathway. Cell Rep Med. 2024;5(2). doi:10.1016/J.XCRM.2024.101401

13. 	García-Vega D, Sánchez-López D, Rodríguez-Carnero G, et al. Semaglutide modulates prothrombotic and atherosclerotic mechanisms, associated with epicardial fat, neutrophils and endothelial cells network. Cardiovasc Diabetol. 2024;23(1). doi:10.1186/S12933-023-02096-9

14. 	Ortea I, Ruiz-Sánchez I, Cañete R, Caballero-Villarraso J, Cañete MD. Identification of candidate serum biomarkers of childhood-onset growth hormone deficiency using SWATH-MS and feature selection. J Proteomics. 2018;175(September 2017):105-113. doi:10.1016/j.jprot.2018.01.003

15. 	Meyer JG, Schilling B. Clinical applications of quantitative proteomics using targeted and untargeted data-independent acquisition techniques. Expert Rev Proteomics. 2017;14(5):419-429. doi:10.1080/14789450.2017.1322904

16. 	Luo Y, Mok TS, Lin X, et al. SWATH-based proteomics identified carbonic anhydrase 2 as a potential diagnosis biomarker for nasopharyngeal carcinoma. Sci Rep. 2017;7(October 2016):1-11. doi:10.1038/srep41191

17. 	Tan HT, Chung MCM. Label-Free Quantitative Phosphoproteomics Reveals Regulation of Vasodilator-Stimulated Phosphoprotein upon Stathmin-1 Silencing in a Pair of Isogenic Colorectal Cancer Cell Lines. Proteomics. 2018;18(8):1-30. doi:10.1002/pmic.201700242